#########################################################################

#

#	Implementation of an MC6809 Simulator in C++

#

#	(C) R.P.Bellis 1993

#

#########################################################################

I recently decided to build a microprocessor based controller,

primarily to revise all of those techniques I learnt as a teenager on

various 8 bit micros, and which aren't so readily practised on today's

systems.  I had a 6809 microprocessor (used in the Dragon 32 and some

Tandy computers) laying around and I managed to pick up a 40*2

character LCD display module from Greenweld Electronics for 9.50

At this point, I decided that it might be useful to have a machine

simulator available before going ahead with any real hardware design.

By doing this I could test the proposed functionality of a system

before going through the rigours of building an entire computer using

prototype wiring techniques.

Using Linux as a development system, I decided on C++ (using the Free

Software Foundation's GCC) rather than C since I saw from the start

that the use of reference variables and derived classes would be of

use.  I don't have room to describe anything like the full

implementation in this article, so I shall describe some important

implementation details which use nice features of C++.

(1) The `mc6809' Base Class

All of the processor registers and the functions that implement the

individual instructions and other pertinent functions have private

access.  Public member functions are provided to load IntelHex files

into the processor memory and to execute instructions.

A simple system which provides no I/O ports (and hence would be of

limited usefulness) could be implemented thus:

#include "mc6809.h"

void main()

{

mc6809		sys;	// Declare the object

sys.load("hexfile");	// Load an Intel hex file

sys.reset();		// Setup initial registers

sys.run();		// Execute forever more

}

The `run' function repeatedly calls `execute' which interprets one

complete instruction.  `Execute' first performs a memory fetch (at the

program counter) and then uses two large switch statements to find

first the addressing mode used by the instruction and then to call the

function which implements the fine detail of the instruction.

(2) Virtualization of I/O Devices

Virtual (protected) member functions are provided to read and write

processor memory.  The base class provides a 64 KB block of RAM and

functions to access memory words and to fetch bytes from the memory

location pointed to by the program counter.

The two virtual functions `mc6809::read' and `mc6809::write' are

provided to be overridden in order to simulate the required I/O

devices.  This can be done simply by checking if the address being

accessed corresponds to the memory locations where I/O devices are

mapped.  If the address doesn't correspond to an I/O device then the

base classes' memory access function is called explicitly by prefixing

the function name with `mc6809::'.

(3) Use of Reference Variables

A feature common on many microprocessors are registers which can also

be split into `high' and `low' halves.  On the 6809 microprocessor

there are the `A' and `B' accumulators which can be joined together to

form a single 16 bit accumulator `D' with the `A' register forming the

upper 8 bits.

In C and C++ this sort of arrangement can be implemented using `union'

records thus:

class mc6809 {

private:

...

Word		X, Y;

struct {

Word	D;

union	{

Byte	A;

Byte	B;

} byte;

} acc;

};

This produces the required effect, whereby modifcation of either of

the 8 bit accumulators also affects the value in the 16 bit `D'

register and vice versa.  The drawback with this is that it requires cumbersome

structure notation of the form `acc.D' or `acc.byte.A' to access the

accumulators.  In C++ this can be got around by using additional

member reference variables which are set to point to the same

addresses as the relevant parts of the structure:

class mc6809 {

...

Byte&		A;

Byte&		B;

Word&		D;

...

};

To actually set the reference variables pointing to the right place

they must be initialized in the base class constructor using the post

parameter list notation:

mc6809::mc6809() : A(acc.byte.A), B(acc.byte.b), D(acc.D)

{

...

}

Once this is done `mc6809::A' becomes a synonym for

`mc6809::acc.byte.A'.  Although this could be done using macro

substitution this would have the side effect of altering any use of the symbol

`A', rather than just those uses where it's needed by the `mc6809'

class.

(4) Further Use of Reference Variables (if you've got room)

Reference variables also come in useful if you don't want to use

pointers excessively.  There are several types of instruction on the

6809 which use a bit pattern to choose a register to act upon.

Sometimes these registers are used as rvalues, but sometimes it's

useful to have them as lvalues which can be modified.  An example of

this is the `do_postincrement' function which is used to increment

register variables after their values have been used in indexed

addressing modes.  The `refreg' function is used to return a reference

variable to be modified:

Word& mc6809::refreg(Byte post)

{

post = (post & 0x60) >> 5;

switch (post) {

case 0: return x;

case 1: return y;

case 2: return u;

default: return s;

}

}

void mc6809::do_postincrement(Byte post)

{

switch (post & 0x9f) {

case 0x80: refreg(post) += 1; break;

case 0x81: refreg(post) += 2; break;

}

}

Thus simply by returning a reference to a variable rather than a copy

of the variable we can use the returned value as an lvalue as well as

an rvalue, and thus can be used with the `+=' operator.  This could of

course been done using pointers, but it would be quite a lot

more cumbersome.

\{end of list of details}

In the initial (working) version of my simulator a very simple derived

class is used which either reads or writes a character using `getchar'

or `putchar' whenever a memory access occurs at address 0xC000.  The

class constructor function puts the terminal into `raw' mode to bypass

the normal line buffering which occurs on Unix systems and `noecho'

mode so that whatever software is running on the simulator has

responsibilty for echoing characters back at the user.  The terminal

is reset to its original mode on exit within the class destructor to

make sure that users can see what is typed on the terminal when they

return to the shell.

If you would like to get hold of the pre-alpha version of my simulator

it's available as \optional{`usim_011.zip' on this month's disk or as}

/pub/mc6809/usim-0.11.tar.Z by Anonymous FTP from ftp.cns.ox.ac.uk if

you have access to the Internet.